Request for Proposals: Third-Party Verification of Benchmarking Reports

[Building Owner] (the “Owner”) seeks a third-party verifier (the “Verifier”) to conduct third-party verification of energy benchmarking data for a property, or properties, located at [address] (the “Property(ies)”) to aid compliance with Maryland's Building Energy Performance Standards (BEPS).

Proposed Scope of Work (SOW)

Task 1: Project Kickoff

At the start of the project, Verifier will:

Prepare an agenda and proposed project schedule.

Schedule one Project Kickoff call with the Owner to confirm project timeline and discuss details of the scope of work.

Task 2: Initial Data Review

Once Owner has provided the information indicated in the Owner Responsibilities section of this proposal (below), Verifier will:

Review collected data for the Property.

Compare collected documentation and publicly available data to the existing ENERGY STAR Portfolio Manager entry for the Property (if available), including basic property information such as gross floor area, review of property use details, energy meters, and energy consumption data in accordance with State requirements.

If the Verifier identifies significant discrepancies between the collected documentation and the existing Portfolio Manager entry, the Verifier will notify the Owner promptly via email. Significant discrepancies would include, but are not limited to, notable differences in utility meter quantity, meter readings, substantial differences in square footage, inclusion or exclusion of specific building areas (such as parking), misclassification of ENERGY STAR Portfolio Manager property types, or other items as noted by the Verifier. This notification would trigger additional tasks which are not included in this base scope of work. See Addendum for optional tasks.

If identified discrepancies are not significant, develop a list of corrections and adjustments required for the Property’s Portfolio Manager entry to align it with confirmed Property data. Provide this list of corrections to the Owner.

Participate in one (1) conference call or meeting to discuss findings of the data review and corrections that need to be applied to the Property’s Portfolio Manager entry. Owner will then make the corrections in Portfolio Manager (see Owner Responsibilities section of this proposal, below).

Task 3: Completion of Data Verification Forms

Once Owner completes corrections provided in Task 2, the Verifier will:

Review the corrected Portfolio Manager entry for the Property and ensure that recommended changes have been applied by the Owner.

Generate, review, and sign the ENERGY STAR Data Verification Checklist and email a digital copy of the completed checklist to the Owner.

Mark the Property as verified in Portfolio Manager, in accordance with State requirements.

Task 4: Project Recap

Once the Data Verification Checklist has been completed and the Property is marked as “verified,” the Verifier will schedule a 30-minute Project Recap call with the Owner to discuss findings, any further changes made to the Portfolio Manager entries, and next steps for the Property.

Owner Responsibilities

In order to successfully complete this project, the Owner is responsible for the following:

Defining a Project Team at the Project Kickoff including, at a minimum:

  - Owner’s Project Manager and Site Representative, which may be the same or different people

  - Any additional people that must be consulted or informed throughout the data verification process.

Providing supporting documentation:

  - Building drawings, including architectural (layout, as-built), mechanical, plumbing, and electrical drawings, at minimum. Additional documentation to verify gross floor area could include record plats, survey documents, or reports based on BOMA floor measurement standards.

  - A breakdown of floor area by space use type and the supporting documentation used to determine this breakdown.

  - Documented proof of time of occupancy for major spaces (e.g., lease requirements) and space use characteristics (e.g., workstation or computer counts).

  - Utility metering and account information, including aggregate meter lists if applicable. Aggregate meter lists should include the meter number(s) and service address(es).

  - Delivered fuel receipt(s) if any.

  - If requested, photos of onsite utility meters; photos need to clearly be able to identify the meter number and (if applicable) space(s) served by the meter.

  - Two years’ worth of base-building utility bills to understand current usage and expenditures.

Managing the Property in ENERGY STAR Portfolio Manager, including:

  - Sharing access to the Property in Portfolio Manager with the Verifier.

  - Making updates to Portfolio Manager data based on list of corrections and adjustments provided by the Verifier.

  - Retaining and keeping in their records the ENERGY STAR Data Verification Checklist provided by the Verifier for no less than five years following the completion of this scope. Note that this checklist does not need to be provided to MDE unless requested by MDE.

  - Confirming the Verifier has added verification information to the property in Portfolio Manager in accordance with State requirements.

  - Owner may designate a representative such as a property manager or benchmarking provider to manage, and make corrections to, the Property’s Portfolio Manager entry.

Owner is ultimately responsible for accurate benchmarking data in accordance with State requirements.

How to Respond to this RFP

Send proposals to [contact email address] no later than [date].

Include the following in the proposal:

  - Detailed scope of work

  - Preliminary schedule

  - Qualifications, including credentials in accordance with State requirements.

  - Price proposal and breakdown

Property Information

Property address: ___

UBID Number: ___

Building typology: ___

Building size: ___

If applicable, unit count: ___

Occupancy: ___

Meter type and quantity: ___

Note:

This resource is a production of the Building Innovation Hub (the Hub) and the Maryland Energy Administration (MEA)’s Clean Buildings Hub initiative. However, this does not guarantee compliance with the State of Maryland’s requirements and laws. It is, ultimately, the responsibility of the individual property owner and data verifier to complete the verification according to the State’s regulations.